Experiment 4: Demonstration of Network Communication Models (OSI / TCP-IP) Using a Network Simulation Tool

## Aim

To demonstrate network communication models (OSI / TCP-IP) using a network simulation tool.

## Objective

- Understand the layers of the OSI and TCP/IP models.

- Build a small network in a simulation tool.

- Observe how data passes through the layers during communication.

- Relate each observed packet to the correct layer.

## Tool Used

Cisco Packet Tracer, a network simulation tool available free to learners through the Cisco Networking Academy. It lets a user build a network, configure devices and watch packets move in Simulation mode.

## OSI Model

| No. | Layer | Function | Data Unit | Example |
| --- | --- | --- | --- | --- |
| 7 | Application | Network services to user programs | Data | HTTP, DNS |
| 6 | Presentation | Format, encryption, compression | Data | TLS, JPEG |
| 5 | Session | Opens, manages and closes sessions | Data | NetBIOS |
| 4 | Transport | End-to-end delivery and error control | Segment | TCP, UDP |
| 3 | Network | Logical addressing and routing | Packet | IP, ICMP |
| 2 | Data Link | Frame delivery on one link, MAC addressing | Frame | Ethernet, ARP |
| 1 | Physical | Bits sent as signals on the medium | Bits | Cable, Wi-Fi |

## TCP/IP Model Mapped to OSI

| TCP/IP Layer | OSI Layers Covered | Protocols |
| --- | --- | --- |
| Application | Application, Presentation, Session | HTTP, FTP, DNS, SMTP |
| Transport | Transport | TCP, UDP |
| Internet | Network | IP, ICMP |
| Network Access | Data Link, Physical | Ethernet, Wi-Fi, ARP |

## Network Setup

| Device | Name | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC | PC0 | 192.168.1.10 | 255.255.255.0 |
| PC | PC1 | 192.168.1.20 | 255.255.255.0 |
| Server | Server0 | 192.168.1.100 | 255.255.255.0 |
| Switch | Switch0 | Not required | Not required |

## Procedure

Step 1: Build the topology. Place two PCs, one server and one switch; connect them with copper straight-through cables.

Step 2: Assign addresses. Open each device, go to Desktop then IP Configuration and enter the addresses above.

Step 3: Test connectivity. Open the command prompt on PC0 and run the ping command to PC1 and to the server.

Step 4: Enable Simulation mode. Switch from Realtime to Simulation mode and filter the events to ICMP, ARP and HTTP.

Step 5: Send traffic. Ping PC1 from PC0, then open the server web page from PC0 in the web browser.

Step 6: Inspect the packets. Click each packet event and open the OSI Model tab to see the layers used at each device.

## Commands Used

ipconfig

ping 192.168.1.20

ping 192.168.1.100

## Data Flow (Encapsulation)

Data  →  Segment (Transport)  →  Packet (Network)  →  Frame (Data Link)  →  Bits (Physical)

At the sender each layer adds its own header. At the receiver the headers are removed in the reverse order (decapsulation) until the data reaches the application.

## Observation Table

| Event | Protocol | Layers Involved at Switch | Data Unit |
| --- | --- | --- | --- |
| PC0 asks who has 192.168.1.20 | ARP | Physical, Data Link | Frame |
| Ping request and reply | ICMP | Physical, Data Link, Network | Packet |
| Web page request | HTTP over TCP | All layers at PC and server | Segment |
| TCP connection set-up | TCP | Transport upward at PC and server | Segment |

## Key Observations

- A switch works at the Data Link layer and forwards frames using MAC addresses.

- ARP finds the MAC address that belongs to an IP address before the first ping.

- HTTP traffic uses TCP, which sets up a connection before sending data.

- The OSI model has seven layers; TCP/IP combines them into four.

## Protocols Used in the Experiment

| Protocol | Layer | Role in the Experiment |
| --- | --- | --- |
| HTTP | Application | Requests the web page from the server |
| TCP | Transport | Sets up a reliable connection for HTTP |
| ICMP | Network | Carries ping request and reply messages |
| IP | Network | Addresses and routes each packet |
| ARP | Data Link | Finds the MAC address for an IP address |
| Ethernet | Data Link / Physical | Sends frames over the cable |

## PDU Names by Layer

- Application, Presentation, Session: data.

- Transport: segment (TCP) or datagram (UDP).

- Network: packet.

- Data Link: frame.

- Physical: bits.

## Comparison of OSI and TCP/IP Models

| Point | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Number of layers | 7 | 4 |
| Origin | Reference model by ISO | Developed with the internet |
| Use | Teaching and troubleshooting | Real internet communication |
| Approach | Layers clearly separated | Protocol based, layers combined |

## Troubleshooting Using the Layers

| Problem | Likely Layer | Check |
| --- | --- | --- |
| Cable unplugged, no link light | Physical | Cables and ports |
| Wrong MAC or switch fault | Data Link | Switch ports and ARP |
| Ping fails, different subnets | Network | IP address and mask |
| Web page does not open | Application | Server service and URL |

## Advantages of a Layered Model

- Each layer has one clear job, so problems are easier to find.

- Layers can be changed without affecting the others.

- Products from different makers work together through common standards.

- It is an easy way to learn and teach networking.

## Packet Tracer Features Used

| Feature | Use |
| --- | --- |
| Realtime mode | Normal operation and ping tests |
| Simulation mode | Step-by-step packet movement |
| PDU information window | Shows layer details of each packet |
| Event list and filters | Shows only ARP, ICMP or HTTP events |

## Learning Outcomes

- Name the layers of the OSI and TCP/IP models and their functions.

- Configure IP addresses and test connectivity with ping.

- Use Simulation mode to trace packets.

- Explain encapsulation and decapsulation.

## Precautions

- Use straight-through cables between unlike devices.

- Check that all devices use the same subnet.

- Save the simulation file before closing the tool.

## Viva Questions

- Q: How many layers does the OSI model have? A: Seven.

- Q: Which layer does a switch work at? A: The Data Link layer.

- Q: What does ARP do? A: It maps an IP address to a MAC address.

- Q: What is encapsulation? A: Adding a header at each layer as data moves down the stack.

## Result

A network was built in Cisco Packet Tracer and the packets were traced layer by layer, confirming the OSI and TCP/IP models in practice.

## Conclusion

Simulation shows how data is encapsulated, transmitted and decapsulated across the layers. Understanding the layers makes it easier to locate faults and to learn how real networks communicate.